| EX NO: 10 | CALCULATION OF PROCESSING GAIN AND GENERATION OF PN SEQUENCES FOR SPREAD SPECTRUM COMMUNICATION USING MATLAB SIMULATION |
| --- | --- |
| Date: |  |

## Objectives

### i) To calculate the processing gain for various PN sequence lengths and tabulate the corresponding processing gain values.

ii) To visualize and compare the processing gain obtained for different PN sequence lengths using MATLAB plots and analyze the effect of sequence length on spread spectrum performance.

PROGRAM (MATLAB Code)

clc;
 	clear;
 	close all;
	% PN sequence lengths
	PN_length = [7 15 31 63 127 255 511 1023];
 	% Processing Gain (Linear)
	PG_linear = PN_length;
 	% Processing Gain in dB
	PG_dB = 10*log10(PG_linear);
 	% Display table
 	T = table(PN_length',PG_dB',...
 		'VariableNames',{'PN_Length','Processing_Gain_dB'});
 	disp('Processing Gain Table');
 	disp(T);
 	% Plot
 	figure;
 	stem(PN_length,PG_dB,'filled');
 	grid on;
 	xlabel('PN Sequence Length');
 	ylabel('Processing Gain (dB)');
 	title('Spread Spectrum: Processing Gain vs PN Sequence Length');

# Objective 1:

## To calculate the processing gain for various PN sequence lengths and tabulate the corresponding values.

### Procedure

1.     Open MATLAB and create a new script file.

2.     Initialize the MATLAB workspace using clc, clear, and close all.

3.     Define the PN sequence lengths.

4.     Calculate the processing gain in linear scale.

5.     Convert the processing gain into decibels.

6.     Create a table containing PN sequence lengths and corresponding processing gains.

7.     Display the table in the MATLAB command window.

8.     Observe the variation in processing gain for different PN sequence lengths.

OUTPUT:

### Result

The processing gain values for different PN sequence lengths were calculated successfully. It was observed that the processing gain increased as the PN sequence length increased.