| EX NO: 3a | GENERATION OF PHASE MODULATED (PM) SIGNALS AND CALCULATION OF PHASE DEVIATION USING MATLAB SIMULATION |
| --- | --- |
| DATE: |  |

### Objectives:

### i) To generate a Phase Modulated (PM) signal in MATLAB using specified carrier and message signal parameters.

### ii) To observe and analyze the time-domain characteristics of the message signal, carrier signal, and phase modulated signal through MATLAB simulation.

### iii) To calculate the phase deviation of the PM signal using the phase sensitivity constant and message signal amplitude.

### iv) To determine the modulation index of the PM signal and examine its relationship with phase deviation.

### v) To investigate the effect of varying message signal amplitude on the phase deviation and modulation characteristics of the PM signal.

### vi) To compare PM waveforms obtained for different phase deviation values and analyze their modulation behavior using MATLAB.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Parameters

Am = 1;      	% Message Amplitude (V)

fm = 100;    	% Message Frequency (Hz)

Ac = 5;      	% Carrier Amplitude (V)

fc = 1000;   	% Carrier Frequency (Hz)

fs = 20*fc;  	% Sampling Frequency (Hz)

kp = pi/2;   	% Phase Sensitivity (rad/V)

% Time Vector

t = 0:1/fs:4/fm;

% Message Signal

m = Am*sin(2*pi*fm*t);

% Carrier Signal

c = Ac*cos(2*pi*fc*t);

% Phase Modulated Signal

pm = Ac*cos(2*pi*fc*t + kp*m);

% Phase Deviation

phase_dev = kp*Am;

% PM Modulation Index

mp = phase_dev;

% Display Results

fprintf('Message Amplitude = %.2f V\n',Am);

fprintf('Message Frequency = %.0f Hz\n',fm);

fprintf('Carrier Amplitude = %.2f V\n',Ac);

fprintf('Carrier Frequency = %.0f Hz\n',fc);

fprintf('Phase Sensitivity = %.2f rad/V\n',kp);

fprintf('Phase Deviation = %.2f radians\n',phase_dev);

fprintf('PM Modulation Index = %.2f\n',mp);

% Plotting

figure('Name','Phase Modulation and Phase Deviation',...

'NumberTitle','off');

subplot(2,2,1);

plot(t,m,'LineWidth',1.5);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,2,2);

plot(t,c,'LineWidth',1.5);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,2,3);

plot(t,pm,'LineWidth',1.5);

title('Phase Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,2,4);

bar(phase_dev);

title('Phase Deviation');

ylabel('Radians');

set(gca,'XTickLabel',{'PM Signal'});

grid on;

### Procedure:

- Open MATLAB and create a new script file.

- Initialize the MATLAB workspace by clearing all variables, command window contents, and open figure windows using:

MATLABclc; clear; close all;

- Define the message signal amplitude ( ), message frequency ( ), carrier amplitude ( ), carrier frequency ( ), sampling frequency ( ), and phase sensitivity constant ( ).

- Generate the time vector required for simulation using the specified sampling frequency.

- Generate the message signal using a sinusoidal waveform with frequency .

- Generate the carrier signal using a sinusoidal waveform with frequency .

- Generate the Phase Modulated (PM) signal

- Plot the message signal, carrier signal, and phase modulated signal using MATLAB plotting commands.

- Observe the phase variations in the PM waveform corresponding to changes in the message signal amplitude.

- Calculate the phase deviation of the PM signal

- Determine the PM modulation index from the calculated phase deviation.

- Display the values of phase deviation and modulation index in the MATLAB Command Window using the fprintf() function.

- Repeat the simulation for different message signal amplitudes and observe the corresponding change in phase deviation.

- Compare the generated PM waveforms for different phase deviation values.

- Record the observations and verify the relationship between message amplitude, phase deviation, and modulation index.

- Analyze the simulation results and validate the characteristics of Phase Modulation.

OUTPUT:

RESULT

The Phase Modulated (PM) signal was successfully generated and analyzed using MATLAB. The phase deviation was calculated from the phase sensitivity constant and message amplitude.

i) Message Frequency = 100 Hz

ii) Carrier Frequency = 1000 Hz

iii) Phase Sensitivity = 1.57 rad/V

iv) Phase Deviation = 1.57 radians

v) PM Modulation Index = 1.57

vi) The PM waveform exhibited phase variations proportional to the message signal amplitude, verifying the characteristics of Phase Modulation.