| EX NO: 8a | SIMULATION AND ANALYSIS OF BPSK MODULATION USING CONSTELLATION DIAGRAMS IN MATLAB |
| --- | --- |
| DATE: |  |

Objective

i) To generate BPSK symbols from a binary data sequence using phase-shift keying principles.

ii) To Plot and analyze the BPSK signal constellation in the In-Phase (I) and Quadrature (Q) plane.

iii) To investigate the effect of noise on constellation points and evaluate the resulting degradation in symbol detection performance.

Matlab code

clc;

clear;

close all;

%% Objective 1: Generate BPSK symbols

N = 1000;                      	% Number of bits

data = randi([0 1], N, 1);     	% Random binary sequence

% BPSK Mapping

bpsk_symbols = 2*data - 1;

%% Objective 2: Plot BPSK Signal Constellation

figure;

subplot(1,2,1);

scatter(real(bpsk_symbols), imag(bpsk_symbols), ...

30, 'filled');

hold on;

xline(0,'k','LineWidth',1.5);

yline(0,'k','LineWidth',1.5);

grid on;

axis equal;

xlim([-1.5 1.5]);

ylim([-1 1]);

xlabel('In-Phase (I)');

ylabel('Quadrature (Q)');

title('Ideal BPSK Constellation');

%% Objective 3: Add Noise and Evaluate Performance

SNR = 5;                       	% Signal-to-Noise Ratio (dB)

rx_signal = awgn(bpsk_symbols, SNR, 'measured');

subplot(1,2,2);

scatter(real(rx_signal), imag(rx_signal), ...

15, 'filled');

hold on;

xline(0,'k','LineWidth',1.5);

yline(0,'k','LineWidth',1.5);

grid on;

axis equal;

xlim([-2.5 2.5]);

ylim([-2 2]);

xlabel('In-Phase (I)');

ylabel('Quadrature (Q)');

title(['Noisy BPSK Constellation (SNR = ', num2str(SNR), ' dB)']);

%% Symbol Detection

detected_bits = real(rx_signal) > 0;

% Calculate Errors

num_errors = sum(data ~= detected_bits);

BER = num_errors/N;

fprintf('Number of Transmitted Bits : %d\n', N);

fprintf('Number of Bit Errors   	: %d\n', num_errors);

fprintf('Bit Error Rate (BER)   	: %f\n', BER);

Procedure:

Objective 1: Generate BPSK Symbols from a Binary Data Sequence Using Phase-Shift Keying Principles

- Initialize the MATLAB environment and define the number of bits.

- Generate a random binary sequence consisting of 0s and 1s.

- Apply BPSK modulation by mapping binary 0 to −1 and binary 1 to +1.

- Store the generated BPSK symbols for further processing.

- Verify that the symbol sequence contains only two distinct symbol values.

- Prepare the modulated symbols for constellation analysis.

OUTPUT:

Result : The BPSK constellation was successfully plotted in the In-Phase (I) and Quadrature (Q) plane, showing two distinct symbol points corresponding to binary 0 and binary 1 for the given SNR.